Midterm Examination preparation

Part I : MCQ

1 Which topology requires a multipoint connection?

- Mesh

- Star

- Bus

- Ring

2.  In TCP/IP the _______ layer is equivalent to the combined session, presentation, and application layers of the OSI model.

  - Application

  - Network

  - Data link

  - Physical

3.  The physical layer is concerned with the movement of _______ over the physical medium.

  - programs

  - dialogs

  - protocols

  - bits

4. _______ is a process-to-process protocol that adds only port addresses, checksum error control, and length information to the data from the upper layer.

  - TCP

  - UDP

  - IP

  - MAC

5.	Which of the following is one of the components of a data communication system

  - Message

  - Sender

  - Medium

  - All of the above choices are correct

6.	In the TCP/IP protocol suite, which of the following is not an application layer protocol?

  - HTTP

  - FTP

  - IP

  - Telnet

- The data link layer packet in the TCP/IP protocol suite is called

  - A message

  - A Frame

  - A segment or user datagram

  - A datagram

- The Internet Protocol (IP) is _______ protocol

  - A reliable

  - A connection oriented

  - A reliable and connection oriented

  - An unreliable

- __________ can impair a signal

- Attenuation

- Distortion

- Noise

- All of the above choices are correct

10          BNC is a type of connector used in ………. Cabling

- Twisted-pair

- Coaxial

- Fiber-optic

- None of the choices are correct

11          Twisted in a twisted-pair help reduce the ……….

- Length

- Cost

- Noise

- None of the choices are correct

12.	. RJ-45 is a type of connectors used in ____________cabling.

a) 	twisted-pair

b) 	coaxial

c) 	fiber-optic

d) 	None of the choices are correct

13  . Transmission media are usually categorized as ………..

  - Fixed or unfixed

  - Guided or unguided

  - Determinate or indeterminate

  - Metallic or nonmetallic

14. Which protocol is used to resolve IP to MAC addresses?

  - DNS

  - DHCP

  - ARP

  - WEP

15.   Media Access Control can be presented only in a _______________link(s)

  - point-to-point

  - multicast

  - both point-to-point and multicast

  - Non of the choices are correct

16   Communication at the data-link layer is

a)       end to end

b)       node to node

c)       process to process

d)        None of the choices are correct.

17. In the ________ random- access method collision is avoided.

a)       CSMA/CD

b)       ALOHA

c)        CSMA/CA

d)        None of the choices are correct.

    - Standard  Ethernet  data rate

- 10Mbps

- 100Mbps

- 1Gbps

- 10Gbps

    - 10 Gigabit Ethernet operates only in

- Simplex mode

- Half duplex mode

- Full duplex

- none

    - In 10Base2  implementation  the medium length

a)  500m

b)  185m

c)  100m

d)  none

PART II  Fill in the Blanks

- The largest geographic area in a WAN can span is____________

The world

- In a star topology with five computers, we need ____________ links

Five

- The data sent in the network layer in TCP/IP model is called ____________	          Datagram  (Packet)

- In BUS topology, at each end of the bus is a ____________, which absorbs any signal, removing it from the bus.  (terminator)

- The _________ layer enables the users to access the network. (application)

- ____________data have discrete states and take discrete values. (Digital)

- Transmission media lie below the __________ Layer. (physical)

- The IEEE 802.11 FHSS uses ___________modulation. ( FSK)

- BLUETOOTH is a_________ Technology that connects devices in a small area. (wireless LAN)

- Data- link layer of a point- to point link has _____sublayers. (one)

Part III TRUE and FALSE

| 1 | In TCP/IP, a message at the transport layer is encapsulated in a packet at the data link layer. | False |
| --- | --- | --- |
| 2 | In the TCP/IP protocol suite, a port number is the identifier at the transport layer. | True |
| 3 | Currently IEEE is responsible for the management of Internet Domain Names & Addresses | False |
| 4 | A WAN interconnects hosts; a LAN interconnects connecting devices such as switches, routers, or modems. | False |
| 5 | A multipoint connection is one in which more than two specific devices share a single link | True |
| 6 | In CDMA, we use different codes to achieve channelization. | True |
| 7 | In the CSMA/CA random access method collision is not avoided. | False |
| 8 | In FDMA, we use different time slots to achieve channelization. | False |
| 9 | RG rating is used in coaxial cable. | True |
| 10 | CSMA/CD stands for carrier sense multiple access collision detection. | True |

PART IV Short Questions.

If a periodic signal is decomposed into five sine waves with frequencies of 100, 300, 500, 700, and 900 Hz, what is its bandwidth? Draw the spectrum, assuming all components have a maximum amplitude of 10 V.

Solution

Let fh be the highest frequency, fl the lowest frequency, and B the bandwidth. Then

- Name the four basic network topologies, and cite an advantage of each type.

Solution

We give an advantage for each of four network topologies:

a. Mesh: secure

b. Bus: easy installation

c. Star: robust

d. Ring: easy fault isolation

3.	. Distinguish between a point-to-point link and a broadcast link.

Ans

A point-to-point link is dedicated to the two devices connecting at the two ends of the link. A broadcast link shares its capacity between pairs of devices that need to use the link.

    - Draw  the figure which shows several layers of Bluetooth?

Solution

5.      What is the difference between the bandwidth and the throughput?

Answer :

The term bandwidth can be used in two different contexts with two different measuring values: bandwidth in hertz and bandwidth in bits per second. The throughput is a measure of how fast we can actually send data through a network. Although, at first glance, bandwidth in bits per second and throughput seem the same, they are different. A link may have a bandwidth of B bps, but we can only send T bps through this link with T always less than B.