IT210 - Computer Network

Assignment 1

Q 1.	Define the three types of data flow. (1 mark)

Ans:

  - In simplex mode, the communication is unidirectional, as on a one-way street. Only one of the two devices on a link can transmit; the other can only receive.

  - In half-duplex mode, each station can both transmit and receive, but not at the same time. When one device is sending, the other can only receive, and vice versa. The half-duplex mode is like a one-lane road with traffic allowed in both directions.

  - In full-duplex mode (also called duplex), both stations can transmit and receive simultaneously. The full-duplex mode is like a two-way street with traffic flowing in both directions at the same time.

Q 2.	What are the two principles of Protocol Layering?  (1 mark)

Ans:

  - The first principle dictates that if we want bidirectional communication, we need to make each layer so that it is able to perform two opposite tasks, one in each direction.

  - The second principle that we need to follow in protocol layering is that the two objects under each layer at both sites should be identical.

Q 3.	 A sine wave is offset 1/4 cycle with respect to time 0. What is its phase in degrees and radians?  (1 mark)

Ans:

  - 1/4 x 360 = 90ْ = 90 x 2Pi/360 = 1.57 rad

Q 4. List three properties for Infrared waves. (1 mark)

Ans:

  - Waves with frequencies from 300 GHz to 400 THz

  - They are used for short-range communication

  - They cannot penetrate walls