Assignment 4

Deadline: Thursday 13/05/2017 @ 23:59

[Total Mark for this Assignment is 40]

Network Management

IT340

Instructions:

- This Assignment must be submitted on Blackboard via the allocated folder.

- Email submission will not be accepted.

- You are advised to make your work clear and well-presented, marks may be reduced for poor presentation.

- You MUST show all your work.

- Late submission will result in ZERO marks being awarded.

- Identical copy from students or other resources will result in ZERO marks for all involved students.

- Convert this Assignment to PDF just before submission.

Student Details:

| Name: Munirah Alrushud CRN: 21335 |  | ID: 140032004 |
| --- | --- | --- |

10 Marks

Learning Outcome(s):

Instructors: State the Learning Outcome(s) that match this question

Question One

The downstream channel bandwidth for the cable modem is 6 MHz. Calculate the bit rate if the signal is: a. QPSK   b. 64-QAM

QPSK:

Bit rate = bandwidth * bits per symbol

bits per symbol = log2 = 2

Bit rate = 6 * 2 = 12 Mbps

64-QAM:

Bit rate = bandwidth * bits per symbol

bits per symbol = log2 (64) = 6

Bit rate = 6 * 6 = 36 Mbps

10 Marks

Learning Outcome(s):

Instructors: State the Learning Outcome(s) that match this question

Question Two

Discuss the similarities and the differences between Multichannel multipoint distribution service (MMDS) and Local multipoint.

Multichannel multipoint distribution service:

It consists of tall antenna tower, backbone internet connectivity using router and network management system. It has microwave link like architecture. The data rate for MMDS is up to 2 Mbps. The CPE cost and deployment cost lower. Also the distance coverage is 50 – 100 km.

Local multipoint distribution service:

It consists of NOC, CPE, BS and Fiber backbone. It has cellular like architecture. The data rate for LMDS is 1 – 10 Mbps. The CPE cost and deployment cost is medium to high. And the distance coverage is 5 – 10 km.

The similarities are that they are both implemented using cable modem at both ends. Also, the network topology for both of them is point to multipoint.

Learning Outcome(s):

Instructors: State the Learning Outcome(s) that match this question

10 Marks

Question Three

Draw and explain HAVi protocol architecture.

Application/service HAVi

TCP/IP

IEEE1394

Application for home entertainment and AV devices

HAVi Components

- Device

- FAV (Full Audio Visual)

- Intermediate AV

- Base AV

- Legacy AV

- Device control module (DCM): Aggregate of FCMs

- Functional control module (FCM): Controls application functions

- Peer-to-peer environment

10 Marks

Learning Outcome(s):

Instructors: State the Learning Outcome(s) that match this question

Question Four (Bonus)

Highlight the differences between the features of traditional networks and next generation networks.

The traditional network:

A traditional network management based. In the application layer. pre-determined routing of calls based on dialed. Has different Services for different platforms.

Traditional network each network has its feature and services:

- The private network has a security.

- The Internet network has a flexibility.

- GSM network has a mobility.

- Cable and television network has content richness.

- The ATM network has a control.

- The optical network has bandwidth.

- The public switched telephone network has reliability.

The next generation network:

A packet based network, it’s able to provide services such as telecommunication services and to make use of multiple broadband. In the application layer - management layer. Dynamic policy based traffic routing. Has multiple Services (Merge phone network, mobile network, internet) in one IP packet network

Next Generational Network has full features:

- GSM's mobility

- Private network's security

- Content richness of cable/television

- Internet flexibility

- Ease of Ethernet

- ATM control

- Bandwidth of optical network

- Reliability of PSTN